# Puskás Tivadar Egyetem

# Hálózati eszközök konfigurációs beállításai

HBONE-RTR

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname HBONE-RTR

!

!

!

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

username HBONE-RTR password 7 0822455D0A16

username TanuloRTR password 7 0822455D0A16

!

!

!

!

!

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name routing.hbone.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0/0

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/0/1

description Web fele

ip address 94.199.49.94 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/0/2

description PTE WAN fele

ip address 209.165.1.2 255.255.255.248

duplex auto

speed auto

!

interface Serial0/2/0

description TanuloRTR fele

ip address 3.255.255.1 255.255.255.252

encapsulation ppp

ppp authentication pap

ppp pap sent-username TanuloRTR password 0 cisco

clock rate 4000000

!

interface Serial0/2/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 0.0.0.0 0.0.0.0 209.165.1.1

ip route 0.0.0.0 0.0.0.0 3.255.255.2

ip route 192.168.0.0 255.255.255.0 3.255.255.2

!

ip flow-export version 9

!

!

!

no cdp run

!

banner motd ^CHBONE Edge Router-re csatlakozott! Illetektelen hozzaferes torvenyellenes!^C

!

!

!

!

line con 0

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

end

TanuloRTR

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname TanuloRTR

!

!

!

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

ip dhcp excluded-address 192.168.0.254

!

ip dhcp pool DHCPPOOL

network 192.168.0.0 255.255.255.0

default-router 192.168.0.254

dns-server 8.8.8.8

domain-name t.hu

!

!

!

no ip cef

no ipv6 cef

!

!

!

username HBONE-RTR password 7 0822455D0A16

username TanuloRTR password 7 0822455D0A16

!

!

license udi pid CISCO1941/K9 sn FTX15249KL6-

license boot module c1900 technology-package securityk9

!

!

!

crypto isakmp policy 1

encr aes

hash md5

authentication pre-share

group 5

!

crypto isakmp key cisco address 209.165.1.1

!

!

!

crypto ipsec transform-set transet esp-aes esp-md5-hmac

!

crypto map vpnmap1 1 ipsec-isakmp

set peer 209.165.1.1

set transform-set transet

match address 100

!

!

!

!

no ip domain-lookup

ip domain-name dsl.pool.telekom.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Tunnel1

ip address 10.20.30.2 255.255.255.252

mtu 1476

tunnel source Serial0/0/0

tunnel destination 209.165.1.1

!

!

interface GigabitEthernet0/0

description Kliens fele

ip address 192.168.0.254 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

description Telekom fele

ip address 3.255.255.2 255.255.255.252

encapsulation ppp

ppp authentication pap

ppp pap sent-username HBONE-RTR password 0 cisco

crypto map vpnmap1

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router rip

!

ip classless

ip route 0.0.0.0 0.0.0.0 Serial0/0/0

ip route 192.168.1.0 255.255.255.240 10.20.30.1

!

ip flow-export version 9

!

!

access-list 100 permit ip 192.168.0.0 0.0.0.255 192.168.1.0 0.0.0.15

!

no cdp run

!

banner motd ^CTelekom CPE-re csatlakozott! Illetektelen hozzaferes torvenyellenes!^C

!

!

!

!

line con 0

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login

line vty 5 15

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login

!

!

!

end

Edge

version 15.4

service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname Edge

!

login block-for 120 attempts 3 within 60

!

!

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

aaa new-model

!

aaa authentication login belep local

!

!

aaa authorization network diakvpn local

!

!

!

!

!

ip cef

no ipv6 cef

!

!

!

username diakdiak password 0 diakdiak

!

!

crypto isakmp policy 1

encr aes

hash md5

authentication pre-share

group 5

!

crypto isakmp key cisco address 3.255.255.2

!

!

crypto isakmp client configuration group diakvpn

key diakdiak

pool VPNP

!

!

crypto ipsec transform-set tset esp-aes esp-md5-hmac

crypto ipsec transform-set transet esp-aes esp-md5-hmac

!

crypto map vpnmap1 1 ipsec-isakmp

set peer 3.255.255.2

set transform-set transet

match address 100

!

!

!

!

no ip domain-lookup

ip domain-name core.ptuni.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Tunnel1

ip address 10.20.30.1 255.255.255.252

mtu 1476

tunnel source GigabitEthernet0/0/1

tunnel destination 3.255.255.2

!

!

interface GigabitEthernet0/0/0

ip address 10.1.1.6 255.255.255.252

ip nat inside

duplex auto

speed auto

!

interface GigabitEthernet0/0/1

ip address 209.165.1.1 255.255.255.248

ip nat outside

duplex auto

speed auto

crypto map vpnmap1

!

interface GigabitEthernet0/0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 10

router-id 5.5.5.4

log-adjacency-changes

passive-interface GigabitEthernet0/0/1

network 10.1.1.4 0.0.0.3 area 0

network 209.165.1.0 0.0.0.7 area 0

default-information originate

!

ip nat pool NATPOOL 209.165.1.5 209.165.1.6 netmask 255.255.255.252

ip nat inside source list LANPOOL pool NATPOOL overload

ip nat inside source static 192.168.1.2 209.165.1.3

ip nat inside source static 192.168.1.10 209.165.1.4

ip classless

ip route 0.0.0.0 0.0.0.0 209.165.1.2

ip route 192.168.0.0 255.255.255.0 10.20.30.2

!

ip flow-export version 9

!

!

ip access-list extended sl_def_acl

deny tcp any any eq telnet

deny tcp any any eq www

deny tcp any any eq 22

permit tcp any any eq 22

ip access-list standard LANPOOL

permit 172.16.0.0 0.0.255.255

access-list 100 permit ip 192.168.1.0 0.0.0.15 192.168.0.0 0.0.0.255

!

no cdp run

!

banner motd ^CEz a router a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

!

!

!

snmp-server community PTE RW

!

logging 192.168.1.10

line con 0

!

line aux 0

!

line vty 0 4

!

!

!

end

ASA0

ASA Version 9.6(1)

!

hostname PTE-ASA

domain-name core.ptuni.hu

enable password aXKtwTDyn2IXb.FS encrypted

names

!

interface GigabitEthernet1/1

description PTE-RTR fele

nameif PTE-IN

security-level 100

ip address 10.1.1.2 255.255.255.252

!

interface GigabitEthernet1/2

description Edge fele

nameif PTE-OUT

security-level 0

ip address 10.1.1.5 255.255.255.252

!

interface GigabitEthernet1/3

no nameif

no security-level

no ip address

shutdown

!

interface GigabitEthernet1/4

no nameif

no security-level

no ip address

shutdown

!

interface GigabitEthernet1/5

no nameif

no security-level

no ip address

shutdown

!

interface GigabitEthernet1/6

no nameif

no security-level

no ip address

shutdown

!

interface GigabitEthernet1/7

no nameif

no security-level

no ip address

shutdown

!

interface GigabitEthernet1/8

no nameif

no security-level

no ip address

shutdown

!

interface Management1/1

management-only

no nameif

no security-level

no ip address

shutdown

!

object network PTE-CORE

subnet 10.0.0.0 255.254.0.0

object network PTE-LAN

subnet 172.16.0.0 255.240.0.0

object network PTE-S01

host 192.168.1.2

object network PTE-S02

host 192.168.1.10

object network PTE-VPNUSER

subnet 192.168.0.0 255.255.255.0

!

!

access-list 200 extended permit icmp any object PTE-LAN echo-reply

access-list 200 extended permit icmp any object PTE-CORE echo-reply

access-list 200 extended permit tcp any object PTE-S02 eq www

access-list 200 extended permit tcp any object PTE-S02 eq 443

access-list 200 extended permit icmp object PTE-VPNUSER object PTE-LAN echo

access-list 100 extended permit tcp any eq www any

access-list 100 extended permit tcp any eq 443 any

!

!

access-group 200 out interface PTE-IN

access-group 100 in interface PTE-OUT

!

username Admin1 password XK9CbN4MkQEI0jdt encrypted

username Admin2 password XK9CbN4MkQEI0jdt encrypted

!

class-map inspection_default

match default-inspection-traffic

!

policy-map type inspect dns preset_dns_map

parameters

message-length maximum 512

policy-map global_policy

class inspection_default

inspect dns preset_dns_map

inspect ftp

inspect tftp

!

service-policy global_policy global

!

telnet timeout 5

ssh timeout 5

!

!

!

!

router ospf 10

router-id 1.1.1.2

log-adjacency-changes

network 10.1.1.0 255.255.255.252 area 0

network 10.1.1.4 255.255.255.252 area 0

PTE-RTR

Building configuration...

Current configuration : 3293 bytes

!

version 16.3.2

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname PTE-RTR

!

!

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

no ip cef

ip routing

!

no ipv6 cef

!

!

!

username Admin1 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

!

!

!

!

ip ftp username PTE-RTR

ip ftp password cisco123

ip ssh version 2

no ip domain-lookup

ip domain-name core.ptuni.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet1/0/1

no switchport

no ip address

duplex auto

speed auto

!

interface GigabitEthernet1/0/2

no switchport

ip address 192.168.1.1 255.255.255.248

duplex auto

speed auto

!

interface GigabitEthernet1/0/3

no switchport

ip address 192.168.1.9 255.255.255.248

duplex auto

speed auto

!

interface GigabitEthernet1/0/4

no switchport

ip address 10.1.1.1 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet1/0/5

no switchport

ip address 10.0.0.25 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet1/0/6

no switchport

ip address 10.0.0.50 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet1/0/7

no switchport

ip address 10.0.0.1 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet1/0/8

!

interface GigabitEthernet1/0/9

!

interface GigabitEthernet1/0/10

!

interface GigabitEthernet1/0/11

!

interface GigabitEthernet1/0/12

!

interface GigabitEthernet1/0/13

!

interface GigabitEthernet1/0/14

!

interface GigabitEthernet1/0/15

!

interface GigabitEthernet1/0/16

!

interface GigabitEthernet1/0/17

!

interface GigabitEthernet1/0/18

!

interface GigabitEthernet1/0/19

!

interface GigabitEthernet1/0/20

!

interface GigabitEthernet1/0/21

!

interface GigabitEthernet1/0/22

!

interface GigabitEthernet1/0/23

!

interface GigabitEthernet1/0/24

!

interface GigabitEthernet1/1/1

!

interface GigabitEthernet1/1/2

!

interface GigabitEthernet1/1/3

!

interface GigabitEthernet1/1/4

!

interface Vlan1

no ip address

shutdown

!

router ospf 10

router-id 1.1.1.1

log-adjacency-changes

redistribute static

passive-interface GigabitEthernet1/0/1

passive-interface GigabitEthernet1/0/2

passive-interface GigabitEthernet1/0/3

network 10.0.0.24 0.0.0.3 area 0

network 10.0.0.48 0.0.0.3 area 0

network 10.0.0.0 0.0.0.3 area 0

network 10.1.1.0 0.0.0.3 area 0

network 192.168.1.0 0.0.0.7 area 0

network 192.168.1.8 0.0.0.7 area 0

!

ip classless

ip route 172.16.15.0 255.255.255.192 10.0.0.2 120

ip route 172.16.16.128 255.255.255.192 10.0.0.2 120

!

ip flow-export version 9

!

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzafrs torvenyellenes!^C

!

!

!

snmp-server community PTE RW

!

logging 192.168.1.10

line con 0

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login

!

line aux 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

GFF-RTR

Current configuration : 1460 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname GFF-RTR

!

!

!

enable secret 5 $1$mERr$5.a6P4JqbNiMX01usIfka/

!

!

!

!

!

!

ip cef

no ipv6 cef

!

!

!

username HBONEAdmin privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name hbone.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0/0

description GFF-MLS-1 fele

ip address 10.0.0.46 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/0/1

description PTE-RTR fele

ip address 10.0.0.49 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 10

router-id 10.10.10.2

log-adjacency-changes

network 10.0.0.48 0.0.0.3 area 0

network 10.0.0.44 0.0.0.3 area 30

!

ip classless

!

ip flow-export version 9

!

!

!

no cdp run

!

banner motd ^CHBONE Puskas Tivadar Egyetem Gazdasagi Router-re csatlakozott, illetektelen hozzaferes torvenyellenes!^C

!

!

!

!

line con 0

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

end

GFF-MLS-1

version 16.3.2

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname GFF-MLS-1

!

!

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

no ip cef

ip routing

!

no ipv6 cef

!

!

!

username Admin1 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

!

!

ip arp inspection validate src-mac dst-mac ip

!

ip dhcp snooping vlan 10,20,40,99

ip dhcp snooping

!

ip ftp username GFF-MLS-1

ip ftp password cisco123

ip ssh version 2

no ip domain-lookup

ip domain-name gtk.ptuni.hu

!

!

spanning-tree mode pvst

!

class-map match-all TANARISUBNET

match access-group 100

!

policy-map TANAR-QOS

class TANARISUBNET

priority percent 25

set precedence 5

!

!

!

!

!

interface GigabitEthernet1/0/1

no switchport

ip address 10.0.0.45 255.255.255.252

service-policy output TANAR-QOS

duplex auto

speed auto

!

interface GigabitEthernet1/0/2

description GFF-B-1 fele

ip dhcp snooping limit rate 5

switchport access vlan 99

switchport mode access

switchport nonegotiate

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0001.4284.1248

spanning-tree portfast

spanning-tree bpduguard enable

!

interface GigabitEthernet1/0/3

description GFF-BAP fele

ip dhcp snooping limit rate 120

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

switchport nonegotiate

!

interface GigabitEthernet1/0/4

description GFF-GT01SW fele

ip dhcp snooping trust

ip dhcp snooping limit rate 135

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

switchport nonegotiate

!

interface GigabitEthernet1/0/5

description GFF-GT02SW fele

ip dhcp snooping trust

ip dhcp snooping limit rate 15

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

switchport nonegotiate

!

interface GigabitEthernet1/0/6

description GFF-GT03SW fele

ip dhcp snooping trust

ip dhcp snooping limit rate 135

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

switchport nonegotiate

!

interface GigabitEthernet1/0/7

description GFF-GT04SW fele

ip dhcp snooping trust

ip dhcp snooping limit rate 135

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

switchport nonegotiate

!

interface GigabitEthernet1/0/8

description GFF-ESW fele

ip dhcp snooping trust

ip dhcp snooping limit rate 240

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

switchport nonegotiate

!

interface GigabitEthernet1/0/9

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/0/10

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/0/11

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/0/12

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/0/13

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/0/14

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/0/15

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/0/16

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/0/17

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/0/18

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/0/19

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/0/20

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/0/21

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/0/22

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/0/23

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/0/24

description GFF-WLC fele

ip dhcp snooping trust

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

switchport nonegotiate

spanning-tree portfast trunk

!

interface GigabitEthernet1/1/1

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/1/2

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/1/3

description Nem hasznalt port

shutdown

!

interface GigabitEthernet1/1/4

description Nem hasznalt port

shutdown

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

description Diak SVI

mac-address 000c.8592.5901

ip address 172.16.8.1 255.255.254.0

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

ip access-group DIAKHARDENING in

!

interface Vlan20

description Tanar SVI

mac-address 000c.8592.5902

ip address 172.16.18.1 255.255.255.224

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

!

interface Vlan40

description Guest SVI

mac-address 000c.8592.5903

ip address 172.16.14.1 255.255.255.0

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

ip access-group GUESTHARDENING in

!

interface Vlan99

description Management+Native SVI

mac-address 000c.8592.5904

ip address 172.16.17.65 255.255.255.192

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

!

router ospf 10

router-id 10.10.10.1

log-adjacency-changes

passive-interface GigabitEthernet1/0/2

passive-interface GigabitEthernet1/0/3

passive-interface GigabitEthernet1/0/4

passive-interface GigabitEthernet1/0/5

passive-interface GigabitEthernet1/0/6

passive-interface GigabitEthernet1/0/7

passive-interface GigabitEthernet1/0/8

network 10.0.0.44 0.0.0.3 area 30

network 172.16.8.0 0.0.1.255 area 30

network 172.16.18.0 0.0.0.31 area 30

network 172.16.14.0 0.0.0.255 area 30

network 172.16.17.64 0.0.0.63 area 30

!

ip classless

!

ip flow-export version 9

!

!

ip access-list extended DIAKHARDENING

remark csak tanar es management es szerver fele mehet kommunikacio, icmp echo tilt

permit ip any host 192.168.1.2

permit ip any host 192.168.1.10

deny icmp any any echo

deny ip any 172.16.16.0 0.0.1.255

deny ip any 172.16.17.128 0.0.0.31

deny ip any 172.16.10.0 0.0.0.255

deny ip any 172.16.2.0 0.0.1.255

deny ip any 172.16.17.160 0.0.0.31

deny ip any 172.16.15.0 0.0.0.63

deny ip any 172.16.12.0 0.0.0.255

deny ip any 172.16.4.0 0.0.1.255

deny ip any 172.16.6.0 0.0.1.255

deny ip any 172.16.17.224 0.0.0.31

deny ip any 172.16.13.0 0.0.0.255

deny ip any 172.16.18.0 0.0.0.31

deny ip any 172.16.14.0 0.0.0.255

permit ip any any

ip access-list extended GUESTHARDENING

remark csak szerver, management es net, szervert se pingelhet

deny icmp any any echo

permit ip any host 192.168.1.2

permit ip any host 192.168.1.10

deny ip any 172.16.0.0 0.0.1.255

deny ip any 172.16.17.128 0.0.0.31

deny ip any 172.16.2.0 0.0.1.255

deny ip any 172.16.17.160 0.0.0.31

deny ip any 172.16.15.0 0.0.0.63

deny ip any 172.16.11.0 0.0.0.255

deny ip any 172.16.4.0 0.0.1.255

deny ip any 172.16.17.192 0.0.0.31

deny ip any 172.16.6.0 0.0.1.255

deny ip any 172.16.17.224 0.0.0.31

deny ip any 172.16.13.0 0.0.0.255

deny ip any 172.16.8.0 0.0.1.255

deny ip any 172.16.18.0 0.0.0.31

permit ip any any

access-list 100 permit ip 172.16.18.0 0.0.0.31 any

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzafrs torvenyellenes!^C

!

!

!

snmp-server community PTE RW

!

logging 192.168.1.10

line con 0

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

GFF-ESW

Current configuration : 3729 bytes

!

version 15.0

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname GFF-ESW

!

ip ftp username GFF-ESW

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name gtk.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

ip arp inspection validate src-mac dst-mac ip

!

ip dhcp snooping vlan 10,20,40,99

ip dhcp snooping

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

description GFF-EAP1 fele

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip dhcp snooping limit rate 240

switchport mode trunk

switchport nonegotiate

spanning-tree portfast trunk

!

interface FastEthernet0/2

description GFF-EAP2 fele

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip dhcp snooping limit rate 240

switchport mode trunk

switchport nonegotiate

spanning-tree portfast trunk

!

interface FastEthernet0/3

description Nem hasznalt port

shutdown

!

interface FastEthernet0/4

description Nem hasznalt port

shutdown

!

interface FastEthernet0/5

description Nem hasznalt port

shutdown

!

interface FastEthernet0/6

description Nem hasznalt port

shutdown

!

interface FastEthernet0/7

description Nem hasznalt port

shutdown

!

interface FastEthernet0/8

description Nem hasznalt port

shutdown

!

interface FastEthernet0/9

description Nem hasznalt port

shutdown

!

interface FastEthernet0/10

description Nem hasznalt port

shutdown

!

interface FastEthernet0/11

description Nem hasznalt port

shutdown

!

interface FastEthernet0/12

description Nem hasznalt port

shutdown

!

interface FastEthernet0/13

description Nem hasznalt port

shutdown

!

interface FastEthernet0/14

description Nem hasznalt port

shutdown

!

interface FastEthernet0/15

description Nem hasznalt port

shutdown

!

interface FastEthernet0/16

description Nem hasznalt port

shutdown

!

interface FastEthernet0/17

description Nem hasznalt port

shutdown

!

interface FastEthernet0/18

description Nem hasznalt port

shutdown

!

interface FastEthernet0/19

description Nem hasznalt port

shutdown

!

interface FastEthernet0/20

description Nem hasznalt port

shutdown

!

interface FastEthernet0/21

description Nem hasznalt port

shutdown

!

interface FastEthernet0/22

description Nem hasznalt port

shutdown

!

interface FastEthernet0/23

description Nem hasznalt port

shutdown

!

interface FastEthernet0/24

description Nem hasznalt port

shutdown

!

interface GigabitEthernet0/1

description GFF-MLS-1 fele

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

switchport nonegotiate

!

interface GigabitEthernet0/2

description Nem hasznalt port

shutdown

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

description Management interface

ip address 172.16.17.74 255.255.255.192

!

ip default-gateway 172.16.17.65

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzafrs torvenyellenes!^C

logging 192.168.1.10

!

!

snmp-server community PTE RW

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

GFF-GT04SW

Current configuration : 4520 bytes

!

version 15.0

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname GFF-GT04SW

!

ip ftp username GFF-GT04SW

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name gtk.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

ip arp inspection validate src-mac dst-mac ip

!

ip dhcp snooping vlan 10,20,40,99

ip dhcp snooping

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

description GFF-GT04-1 fele

switchport access vlan 10

ip dhcp snooping limit rate 5

switchport mode access

switchport nonegotiate

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 000C.85D9.741B

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/2

description GFF-GT04-N1 fele

switchport access vlan 20

ip dhcp snooping limit rate 5

switchport mode access

switchport nonegotiate

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0002.1761.0C86

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/3

description GFF-GT04-2 fele

switchport access vlan 20

ip dhcp snooping limit rate 5

switchport mode access

switchport nonegotiate

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0010.1148.4697

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/4

description GFF-GT04AP fele

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip dhcp snooping limit rate 120

switchport mode trunk

switchport nonegotiate

spanning-tree portfast trunk

!

interface FastEthernet0/5

description Nem hasznalt port

shutdown

!

interface FastEthernet0/6

description Nem hasznalt port

shutdown

!

interface FastEthernet0/7

description Nem hasznalt port

shutdown

!

interface FastEthernet0/8

description Nem hasznalt port

shutdown

!

interface FastEthernet0/9

description Nem hasznalt port

shutdown

!

interface FastEthernet0/10

description Nem hasznalt port

shutdown

!

interface FastEthernet0/11

description Nem hasznalt port

shutdown

!

interface FastEthernet0/12

description Nem hasznalt port

shutdown

!

interface FastEthernet0/13

description Nem hasznalt port

shutdown

!

interface FastEthernet0/14

description Nem hasznalt port

shutdown

!

interface FastEthernet0/15

description Nem hasznalt port

shutdown

!

interface FastEthernet0/16

description Nem hasznalt port

shutdown

!

interface FastEthernet0/17

description Nem hasznalt port

shutdown

!

interface FastEthernet0/18

description Nem hasznalt port

shutdown

!

interface FastEthernet0/19

description Nem hasznalt port

shutdown

!

interface FastEthernet0/20

description Nem hasznalt port

shutdown

!

interface FastEthernet0/21

description Nem hasznalt port

shutdown

!

interface FastEthernet0/22

description Nem hasznalt port

shutdown

!

interface FastEthernet0/23

description Nem hasznalt port

shutdown

!

interface FastEthernet0/24

description Nem hasznalt port

shutdown

!

interface GigabitEthernet0/1

description GFF-MLS-1 fele

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

switchport nonegotiate

!

interface GigabitEthernet0/2

description Nem hasznalt port

shutdown

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

description Management interface

ip address 172.16.17.73 255.255.255.192

!

ip default-gateway 172.16.17.65

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzafrs torvenyellenes!^C

logging 192.168.1.10

!

!

snmp-server community PTE RW

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

GFF-GT03SW

Building configuration...

Current configuration : 4520 bytes

!

version 15.0

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname GFF-GT03SW

!

ip ftp username GFF-GT03SW

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name gtk.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

ip arp inspection validate src-mac dst-mac ip

!

ip dhcp snooping vlan 10,20,40,99

ip dhcp snooping

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

description GFF-GT03-1 fele

switchport access vlan 10

ip dhcp snooping limit rate 5

switchport mode access

switchport nonegotiate

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0003.E44E.769A

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/2

description GFF-GT03-N1 fele

switchport access vlan 20

ip dhcp snooping limit rate 5

switchport mode access

switchport nonegotiate

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0009.7CD6.1DDB

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/3

description GFF-GT03-2 fele

switchport access vlan 20

ip dhcp snooping limit rate 5

switchport mode access

switchport nonegotiate

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 00D0.BCB6.2B13

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/4

description GFF-GT03AP fele

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip dhcp snooping limit rate 120

switchport mode trunk

switchport nonegotiate

spanning-tree portfast trunk

!

interface FastEthernet0/5

description Nem hasznalt port

shutdown

!

interface FastEthernet0/6

description Nem hasznalt port

shutdown

!

interface FastEthernet0/7

description Nem hasznalt port

shutdown

!

interface FastEthernet0/8

description Nem hasznalt port

shutdown

!

interface FastEthernet0/9

description Nem hasznalt port

shutdown

!

interface FastEthernet0/10

description Nem hasznalt port

shutdown

!

interface FastEthernet0/11

description Nem hasznalt port

shutdown

!

interface FastEthernet0/12

description Nem hasznalt port

shutdown

!

interface FastEthernet0/13

description Nem hasznalt port

shutdown

!

interface FastEthernet0/14

description Nem hasznalt port

shutdown

!

interface FastEthernet0/15

description Nem hasznalt port

shutdown

!

interface FastEthernet0/16

description Nem hasznalt port

shutdown

!

interface FastEthernet0/17

description Nem hasznalt port

shutdown

!

interface FastEthernet0/18

description Nem hasznalt port

shutdown

!

interface FastEthernet0/19

description Nem hasznalt port

shutdown

!

interface FastEthernet0/20

description Nem hasznalt port

shutdown

!

interface FastEthernet0/21

description Nem hasznalt port

shutdown

!

interface FastEthernet0/22

description Nem hasznalt port

shutdown

!

interface FastEthernet0/23

description Nem hasznalt port

shutdown

!

interface FastEthernet0/24

description Nem hasznalt port

shutdown

!

interface GigabitEthernet0/1

description GFF-MLS-1 fele

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

switchport nonegotiate

!

interface GigabitEthernet0/2

description Nem hasznalt port

shutdown

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

description Management interface

ip address 172.16.17.72 255.255.255.192

!

ip default-gateway 172.16.17.65

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzafrs torvenyellenes!^C

logging 192.168.1.10

!

!

snmp-server community PTE RW

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

GFF-GT01SW

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname GFF-GT01SW

!

ip ftp username GFF-GT01SW

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name gtk.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

ip arp inspection validate src-mac dst-mac ip

!

ip dhcp snooping vlan 10,20,40,99

ip dhcp snooping

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

description GFF-GT01-1 fele

switchport access vlan 10

ip dhcp snooping limit rate 5

switchport mode access

switchport nonegotiate

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security violation restrict

switchport port-security mac-address sticky 0001.6319.89A6

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/2

description GFF-GT01-N1 fele

switchport access vlan 20

ip dhcp snooping limit rate 5

switchport mode access

switchport nonegotiate

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security violation restrict

switchport port-security mac-address sticky 00D0.5851.7793

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/3

description GFF-GT01-2 fele

switchport access vlan 20

ip dhcp snooping limit rate 5

switchport mode access

switchport nonegotiate

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security violation restrict

switchport port-security mac-address sticky 0090.2156.808D

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/4

description GFF-GT01AP fele

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip dhcp snooping limit rate 120

switchport mode trunk

switchport nonegotiate

spanning-tree portfast trunk

!

interface FastEthernet0/5

description Nem hasznalt port

shutdown

!

interface FastEthernet0/6

description Nem hasznalt port

shutdown

!

interface FastEthernet0/7

description Nem hasznalt port

shutdown

!

interface FastEthernet0/8

description Nem hasznalt port

shutdown

!

interface FastEthernet0/9

description Nem hasznalt port

shutdown

!

interface FastEthernet0/10

description Nem hasznalt port

shutdown

!

interface FastEthernet0/11

description Nem hasznalt port

shutdown

!

interface FastEthernet0/12

description Nem hasznalt port

shutdown

!

interface FastEthernet0/13

description Nem hasznalt port

shutdown

!

interface FastEthernet0/14

description Nem hasznalt port

shutdown

!

interface FastEthernet0/15

description Nem hasznalt port

shutdown

!

interface FastEthernet0/16

description Nem hasznalt port

shutdown

!

interface FastEthernet0/17

description Nem hasznalt port

shutdown

!

interface FastEthernet0/18

description Nem hasznalt port

shutdown

!

interface FastEthernet0/19

description Nem hasznalt port

shutdown

!

interface FastEthernet0/20

description Nem hasznalt port

shutdown

!

interface FastEthernet0/21

description Nem hasznalt port

shutdown

!

interface FastEthernet0/22

description Nem hasznalt port

shutdown

!

interface FastEthernet0/23

description Nem hasznalt port

shutdown

!

interface FastEthernet0/24

description Nem hasznalt port

shutdown

!

interface GigabitEthernet0/1

description GFF-MLS-1 fele

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

switchport nonegotiate

!

interface GigabitEthernet0/2

description Nem hasznalt port

shutdown

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

description Management interface

ip address 172.16.17.70 255.255.255.192

!

ip default-gateway 172.16.17.65

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzafrs torvenyellenes!^C

logging 192.168.1.10

!

!

snmp-server community PTE RW

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

GFF-GT02SW

Building configuration...

Current configuration : 4358 bytes

!

version 15.0

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname GFF-GT02SW

!

ip ftp username GFF-GT02SW

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name gtk.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

ip arp inspection validate src-mac dst-mac ip

!

ip dhcp snooping vlan 10,20,40,99

ip dhcp snooping

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

description GFF-GT02-1 fele

switchport access vlan 10

ip dhcp snooping limit rate 5

switchport mode access

switchport nonegotiate

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0006.2A4E.EE6B

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/2

description GFF-GT02-N1 fele

switchport access vlan 20

ip dhcp snooping limit rate 5

switchport mode access

switchport nonegotiate

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0001.9642.9A67

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/3

description GFF-GT02-T1,GFF-GT02-2 fele

switchport access vlan 20

ip dhcp snooping limit rate 5

switchport mode access

switchport nonegotiate

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0090.2BA1.0C32

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/4

description Nem hasznalt port

shutdown

!

interface FastEthernet0/5

description Nem hasznalt port

shutdown

!

interface FastEthernet0/6

description Nem hasznalt port

shutdown

!

interface FastEthernet0/7

description Nem hasznalt port

shutdown

!

interface FastEthernet0/8

description Nem hasznalt port

shutdown

!

interface FastEthernet0/9

description Nem hasznalt port

shutdown

!

interface FastEthernet0/10

description Nem hasznalt port

shutdown

!

interface FastEthernet0/11

description Nem hasznalt port

shutdown

!

interface FastEthernet0/12

description Nem hasznalt port

shutdown

!

interface FastEthernet0/13

description Nem hasznalt port

shutdown

!

interface FastEthernet0/14

description Nem hasznalt port

shutdown

!

interface FastEthernet0/15

description Nem hasznalt port

shutdown

!

interface FastEthernet0/16

description Nem hasznalt port

shutdown

!

interface FastEthernet0/17

description Nem hasznalt port

shutdown

!

interface FastEthernet0/18

description Nem hasznalt port

shutdown

!

interface FastEthernet0/19

description Nem hasznalt port

shutdown

!

interface FastEthernet0/20

description Nem hasznalt port

shutdown

!

interface FastEthernet0/21

description Nem hasznalt port

shutdown

!

interface FastEthernet0/22

description Nem hasznalt port

shutdown

!

interface FastEthernet0/23

description Nem hasznalt port

shutdown

!

interface FastEthernet0/24

description Nem hasznalt port

shutdown

!

interface GigabitEthernet0/1

description GFF-MLS-1 fele

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

switchport nonegotiate

!

interface GigabitEthernet0/2

description Nem hasznalt port

shutdown

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

description Management interface

ip address 172.16.17.71 255.255.255.192

!

ip default-gateway 172.16.17.65

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzafrs torvenyellenes!^C

logging 192.168.1.10

!

!

snmp-server community PTE RW

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

end

MFF-RTR

Current configuration : 1948 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname MFF-RTR

!

login block-for 120 attempts 3 within 60

!

!

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

no ip cef

ipv6 unicast-routing

!

no ipv6 cef

!

!

!

username Admin1 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

!

!

ip ftp username MFF-RTR

ip ftp password cisco123

ip ssh version 2

no ip domain-lookup

ip domain-name hbone.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0/0

description MFF-MLS1 fele

ip address 10.0.0.29 255.255.255.252

duplex auto

speed auto

ipv6 address 2002:CAFE:EFAC:3::1/64

ipv6 eigrp 10

!

interface GigabitEthernet0/0/1

description PTE-RTR fele

ip address 10.0.0.26 255.255.255.252

duplex auto

speed auto

ipv6 address 2002:CAFE:EFAC:2::2/64

!

interface GigabitEthernet0/0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 10

router-id 7.7.7.7

log-adjacency-changes

network 10.0.0.24 0.0.0.3 area 0

network 10.0.0.28 0.0.0.3 area 0

!

ipv6 router eigrp 10

eigrp router-id 15.15.15.15

no shutdown

passive-interface GigabitEthernet0/0/1

redistribute static

!

ip classless

!

ip flow-export version 9

!

ipv6 route ::/0 2002:CAFE:EFAC:2::1

!

ip access-list extended sl_def_acl

deny tcp any any eq telnet

deny tcp any any eq www

deny tcp any any eq 22

permit tcp any any eq 22

!

no cdp run

!

banner motd ^CHBone felugyeleti eszkoz^C

!

!

!

!

line con 0

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

End

MFF-MLS-1

Building configuration...

Current configuration : 2922 bytes

!

version 16.3.2

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname MFF-MLS1

!

login block-for 120 attempts 3 within 60

!

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

no ip cef

ip routing

!

ipv6 unicast-routing

!

no ipv6 cef

!

!

!

username Admin1 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

!

!

!

!

ip ftp username MFF-MLS1

ip ftp password cisco123

ip ssh version 2

no ip domain-lookup

ip domain-name muszaki.ptuni.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet1/0/1

no switchport

ip address 10.0.0.30 255.255.255.252

duplex auto

speed auto

ipv6 address 2002:CAFE:EFAC:3::2/64

ipv6 eigrp 10

!

interface GigabitEthernet1/0/2

!

interface GigabitEthernet1/0/3

!

interface GigabitEthernet1/0/4

!

interface GigabitEthernet1/0/5

!

interface GigabitEthernet1/0/6

!

interface GigabitEthernet1/0/7

!

interface GigabitEthernet1/0/8

!

interface GigabitEthernet1/0/9

!

interface GigabitEthernet1/0/10

!

interface GigabitEthernet1/0/11

!

interface GigabitEthernet1/0/12

!

interface GigabitEthernet1/0/13

!

interface GigabitEthernet1/0/14

!

interface GigabitEthernet1/0/15

!

interface GigabitEthernet1/0/16

!

interface GigabitEthernet1/0/17

!

interface GigabitEthernet1/0/18

!

interface GigabitEthernet1/0/19

!

interface GigabitEthernet1/0/20

!

interface GigabitEthernet1/0/21

!

interface GigabitEthernet1/0/22

!

interface GigabitEthernet1/0/23

!

interface GigabitEthernet1/0/24

no switchport

ip address 10.0.0.33 255.255.255.252

duplex auto

speed auto

ipv6 address 2002:CAFE:EFAC:4::1/64

ipv6 eigrp 10

!

interface GigabitEthernet1/1/1

!

interface GigabitEthernet1/1/2

!

interface GigabitEthernet1/1/3

!

interface GigabitEthernet1/1/4

!

interface Vlan1

no ip address

shutdown

!

router ospf 10

router-id 8.8.8.8

log-adjacency-changes

network 10.0.0.28 0.0.0.3 area 0

network 10.0.0.32 0.0.0.3 area 20

!

ipv6 router eigrp 10

eigrp router-id 15.15.15.16

no shutdown

!

ip classless

!

ip flow-export version 9

!

!

ip access-list extended sl_def_acl

deny tcp any any eq telnet

deny tcp any any eq www

deny tcp any any eq 22

permit tcp any any eq 22

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

!

!

!

snmp-server community PTE RW

!

logging 192.168.1.10

line con 0

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login

!

line aux 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

MFF-MLS-2

Building configuration...

Current configuration : 6308 bytes

!

version 16.3.2

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname MFF-MLS2

!

login block-for 120 attempts 3 within 60

!

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

no ip cef

ip routing

!

ipv6 unicast-routing

!

no ipv6 cef

!

!

!

username Admin1 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

!

!

!

!

ip ftp username MFF-MLS2

ip ftp password cisco123

ip ssh version 2

no ip domain-lookup

ip domain-name muszaki.ptuni.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet1/0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

!

interface GigabitEthernet1/0/2

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

!

interface GigabitEthernet1/0/3

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

!

interface GigabitEthernet1/0/4

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

!

interface GigabitEthernet1/0/5

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

!

interface GigabitEthernet1/0/6

!

interface GigabitEthernet1/0/7

!

interface GigabitEthernet1/0/8

!

interface GigabitEthernet1/0/9

!

interface GigabitEthernet1/0/10

!

interface GigabitEthernet1/0/11

!

interface GigabitEthernet1/0/12

!

interface GigabitEthernet1/0/13

!

interface GigabitEthernet1/0/14

!

interface GigabitEthernet1/0/15

!

interface GigabitEthernet1/0/16

!

interface GigabitEthernet1/0/17

!

interface GigabitEthernet1/0/18

!

interface GigabitEthernet1/0/19

!

interface GigabitEthernet1/0/20

!

interface GigabitEthernet1/0/21

!

interface GigabitEthernet1/0/22

description MFF-WLC fele

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

switchport nonegotiate

!

interface GigabitEthernet1/0/23

no switchport

ip address 10.0.0.37 255.255.255.252

duplex auto

speed auto

ipv6 address 2002:CAFE:EFAC:5::1/64

ipv6 eigrp 10

!

interface GigabitEthernet1/0/24

no switchport

ip address 10.0.0.34 255.255.255.252

duplex auto

speed auto

ipv6 address 2002:CAFE:EFAC:4::2/64

ipv6 eigrp 10

!

interface GigabitEthernet1/1/1

!

interface GigabitEthernet1/1/2

!

interface GigabitEthernet1/1/3

!

interface GigabitEthernet1/1/4

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

mac-address 0030.a3ad.1601

ip address 172.16.4.1 255.255.254.0

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

ip access-group DIAKHARDENING in

ipv6 address FE80::1 link-local

ipv6 address 2002:DB8:CAFE:1::1/64

ipv6 eigrp 10

!

interface Vlan20

mac-address 0030.a3ad.1602

ip address 172.16.17.193 255.255.255.224

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

ipv6 address FE80::1 link-local

ipv6 address 2002:DB8:CAFE:2::1/64

ipv6 eigrp 10

!

interface Vlan40

mac-address 0030.a3ad.1603

ip address 172.16.12.1 255.255.255.0

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

ip access-group GUESTHARDENING in

ipv6 address FE80::1 link-local

ipv6 address 2002:DB8:CAFE:3::1/64

ipv6 eigrp 10

!

interface Vlan99

mac-address 0030.a3ad.1604

ip address 172.16.16.193 255.255.255.192

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

ipv6 address FE80::1 link-local

ipv6 address 2002:DB8:CAFE:5::1/64

ipv6 eigrp 10

!

router ospf 10

router-id 9.9.9.9

log-adjacency-changes

network 10.0.0.32 0.0.0.3 area 20

network 10.0.0.36 0.0.0.3 area 20

network 172.16.4.0 0.0.1.255 area 20

network 172.16.17.192 0.0.0.31 area 20

network 172.16.12.0 0.0.0.255 area 20

network 172.16.16.192 0.0.0.63 area 20

!

ipv6 router eigrp 10

eigrp router-id 15.15.15.17

no shutdown

passive-interface GigabitEthernet1/0/1

passive-interface GigabitEthernet1/0/2

passive-interface GigabitEthernet1/0/3

passive-interface GigabitEthernet1/0/4

passive-interface GigabitEthernet1/0/5

!

ip classless

!

ip flow-export version 9

!

!

ip access-list extended sl_def_acl

deny tcp any any eq telnet

deny tcp any any eq www

deny tcp any any eq 22

permit tcp any any eq 22

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

ip access-list extended DIAKHARDENING

remark csak tanar es management es szerver fele mehet kommunikacio, icmp echo tilt

permit ip any host 192.168.1.2

permit ip any host 192.168.1.10

deny icmp any any echo

deny ip any 172.16.16.0 0.0.1.255

deny ip any 172.16.17.128 0.0.0.31

deny ip any 172.16.10.0 0.0.0.255

deny ip any 172.16.2.0 0.0.1.255

deny ip any 172.16.17.160 0.0.0.31

deny ip any 172.16.15.0 0.0.0.63

deny ip any 172.16.12.0 0.0.0.255

deny ip any 172.16.6.0 0.0.1.255

deny ip any 172.16.17.224 0.0.0.31

deny ip any 172.16.13.0 0.0.0.255

deny ip any 172.16.8.0 0.0.1.255

deny ip any 172.16.18.0 0.0.0.31

deny ip any 172.16.14.0 0.0.0.255

permit ip any any

ip access-list extended GUESTHARDENING

remark csak szerver, management es net, szervert se pingelhet

deny icmp any any echo

permit ip any host 192.168.1.2

permit ip any host 192.168.1.10

deny ip any 172.16.0.0 0.0.1.255

deny ip any 172.16.17.128 0.0.0.31

deny ip any 172.16.2.0 0.0.1.255

deny ip any 172.16.17.160 0.0.0.31

deny ip any 172.16.15.0 0.0.0.63

deny ip any 172.16.11.0 0.0.0.255

deny ip any 172.16.4.0 0.0.1.255

deny ip any 172.16.17.192 0.0.0.31

deny ip any 172.16.6.0 0.0.1.255

deny ip any 172.16.17.224 0.0.0.31

deny ip any 172.16.13.0 0.0.0.255

deny ip any 172.16.8.0 0.0.1.255

deny ip any 172.16.18.0 0.0.0.31

deny ip any 172.16.14.0 0.0.0.255

permit ip any any

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

!

!

!

snmp-server community PTE RW

!

logging 192.168.1.10

line con 0

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login

!

line aux 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

MFF-MT05SW

Building configuration...

Current configuration : 2718 bytes

!

version 15.0

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname MFF-MT05SW

!

ip ftp username MFF-MT05SW

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name muszaki.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

ip dhcp snooping

!

spanning-tree mode pvst

spanning-tree portfast default

spanning-tree portfast bpduguard default

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport access vlan 10

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0030.A3C8.45EC

!

interface FastEthernet0/2

switchport access vlan 20

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0010.1141.23E9

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.16.197 255.255.255.192

!

ip default-gateway 172.16.16.193

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

logging 192.168.1.10

!

!

snmp-server community PTE RW

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

MFF-MT06SW

Current configuration : 2718 bytes

!

version 15.0

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname MFF-MT06SW

!

ip ftp username MFF-MT06SW

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name muszaki.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

ip dhcp snooping

!

spanning-tree mode pvst

spanning-tree portfast default

spanning-tree portfast bpduguard default

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport access vlan 10

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0060.7079.1908

!

interface FastEthernet0/2

switchport access vlan 20

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 000B.BED8.E4E0

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.16.198 255.255.255.192

!

ip default-gateway 172.16.16.193

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

logging 192.168.1.10

!

!

snmp-server community PTE RW

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

end

MFF-MT04SW-1

Building configuration...

Current configuration : 3169 bytes

!

version 15.0

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname MFF-MT04SW-1

!

ip ftp username MFF-MT04SW-1

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name muszaki.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

ip dhcp snooping

!

spanning-tree mode rapid-pvst

spanning-tree portfast default

spanning-tree portfast bpduguard default

spanning-tree extend system-id

spanning-tree vlan 10,20,40,99 priority 24576

!

interface FastEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface FastEthernet0/2

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface FastEthernet0/3

switchport access vlan 10

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 000C.CF49.B54E

!

interface FastEthernet0/4

switchport access vlan 20

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0006.2A09.0A90

!

interface FastEthernet0/5

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.16.194 255.255.255.192

!

ip default-gateway 172.16.16.193

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

logging 192.168.1.10

!

!

snmp-server community PTE RW

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

MFF-MT04SW-2

Current configuration : 2922 bytes

!

version 15.0

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname MFF-MT04SW-2

!

ip ftp username MFF-MT04SW-2

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name muszaki.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

ip dhcp snooping

!

spanning-tree mode rapid-pvst

spanning-tree portfast default

spanning-tree portfast bpduguard default

spanning-tree extend system-id

spanning-tree vlan 10,20,40,99 priority 28672

!

interface FastEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface FastEthernet0/2

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface FastEthernet0/3

switchport access vlan 10

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0000.0C6D.4BC5

!

interface FastEthernet0/4

switchport access vlan 20

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 000C.CF75.7038

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.16.195 255.255.255.192

!

ip default-gateway 172.16.16.193

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

logging 192.168.1.10

!

!

snmp-server community PTE RW

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

MFF-MT04SW-3

Current configuration : 2876 bytes

!

version 15.0

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname MFF-MT04SW-3

!

ip ftp username MFF-MT04SW-3

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name muszaki.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

ip dhcp snooping

!

spanning-tree mode rapid-pvst

spanning-tree portfast default

spanning-tree portfast bpduguard default

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface FastEthernet0/2

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface FastEthernet0/3

switchport access vlan 10

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0001.C780.9A07

!

interface FastEthernet0/4

switchport access vlan 20

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 00E0.A3B6.C580

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.16.196 255.255.255.192

!

ip default-gateway 172.16.16.193

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

logging 192.168.1.10

!

!

snmp-server community PTE RW

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

MFF-MLS-3

Building configuration...

Current configuration : 4985 bytes

!

version 16.3.2

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname MFF-MLS3

!

login block-for 120 attempts 3 within 60

!

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

no ip cef

ip routing

!

ipv6 unicast-routing

!

no ipv6 cef

!

!

!

username Admin1 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

!

!

!

!

ip ftp username MFF-MLS3

ip ftp password cisco123

ip ssh version 2

no ip domain-lookup

ip domain-name muszaki.ptuni.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet1/0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

!

interface GigabitEthernet1/0/2

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

!

interface GigabitEthernet1/0/3

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

!

interface GigabitEthernet1/0/4

!

interface GigabitEthernet1/0/5

!

interface GigabitEthernet1/0/6

!

interface GigabitEthernet1/0/7

!

interface GigabitEthernet1/0/8

!

interface GigabitEthernet1/0/9

!

interface GigabitEthernet1/0/10

!

interface GigabitEthernet1/0/11

!

interface GigabitEthernet1/0/12

!

interface GigabitEthernet1/0/13

!

interface GigabitEthernet1/0/14

!

interface GigabitEthernet1/0/15

!

interface GigabitEthernet1/0/16

!

interface GigabitEthernet1/0/17

!

interface GigabitEthernet1/0/18

!

interface GigabitEthernet1/0/19

!

interface GigabitEthernet1/0/20

!

interface GigabitEthernet1/0/21

!

interface GigabitEthernet1/0/22

!

interface GigabitEthernet1/0/23

!

interface GigabitEthernet1/0/24

no switchport

ip address 10.0.0.38 255.255.255.252

duplex auto

speed auto

ipv6 address 2002:CAFE:EFAC:5::2/64

ipv6 eigrp 10

!

interface GigabitEthernet1/1/1

!

interface GigabitEthernet1/1/2

!

interface GigabitEthernet1/1/3

!

interface GigabitEthernet1/1/4

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

mac-address 000c.cf37.2401

ip address 172.16.6.1 255.255.254.0

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

ip access-group DIAKHARDENING in

ipv6 address FE80::1 link-local

ipv6 address 2002:DB8:CAFE:6::1/64

ipv6 eigrp 10

!

interface Vlan20

mac-address 000c.cf37.2402

ip address 172.16.17.225 255.255.255.224

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

ipv6 address FE80::1 link-local

ipv6 address 2002:DB8:CAFE:7::1/64

ipv6 eigrp 10

!

interface Vlan40

mac-address 000c.cf37.2403

ip address 172.16.13.1 255.255.255.0

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

ipv6 address FE80::1 link-local

ipv6 address 2002:DB8:CAFE:8::1/64

ipv6 eigrp 10

!

interface Vlan99

mac-address 000c.cf37.2404

ip address 172.16.17.1 255.255.255.192

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

ipv6 address FE80::1 link-local

ipv6 address 2002:DB8:CAFE:10::1/64

ipv6 eigrp 10

!

router ospf 10

router-id 10.10.10.10

log-adjacency-changes

network 10.0.0.36 0.0.0.3 area 20

network 172.16.6.0 0.0.1.255 area 20

network 172.16.17.224 0.0.0.31 area 20

network 172.16.13.0 0.0.0.255 area 20

network 172.16.17.0 0.0.0.63 area 20

!

ipv6 router eigrp 10

eigrp router-id 15.15.15.18

no shutdown

passive-interface GigabitEthernet1/0/1

passive-interface GigabitEthernet1/0/2

passive-interface GigabitEthernet1/0/3

!

ip classless

!

ip flow-export version 9

!

!

ip access-list extended sl_def_acl

deny tcp any any eq telnet

deny tcp any any eq www

deny tcp any any eq 22

permit tcp any any eq 22

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

ip access-list extended DIAKHARDENING

remark csak tanar es management es szerver fele mehet kommunikacio, icmp echo tilt

permit ip any host 192.168.1.2

permit ip any host 192.168.1.10

deny icmp any any echo

deny ip any 172.16.16.0 0.0.1.255

deny ip any 172.16.17.128 0.0.0.31

deny ip any 172.16.10.0 0.0.0.255

deny ip any 172.16.2.0 0.0.1.255

deny ip any 172.16.17.160 0.0.0.31

deny ip any 172.16.15.0 0.0.0.63

deny ip any 172.16.12.0 0.0.0.255

deny ip any 172.16.4.0 0.0.1.255

deny ip any 172.16.17.224 0.0.0.31

deny ip any 172.16.13.0 0.0.0.255

deny ip any 172.16.8.0 0.0.1.255

deny ip any 172.16.18.0 0.0.0.31

deny ip any 172.16.14.0 0.0.0.255

permit ip any any

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

!

!

!

snmp-server community PTE RW

!

logging 192.168.1.10

line con 0

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login

!

line aux 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

MFF-MT01SW

Current configuration : 2714 bytes

!

version 15.0

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname MFF-MT01SW

!

ip ftp username MFF-MT01SW

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name muszaki.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

ip dhcp snooping

!

spanning-tree mode pvst

spanning-tree portfast default

spanning-tree portfast bpduguard default

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport access vlan 10

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0002.4AD9.7E61

!

interface FastEthernet0/2

switchport access vlan 20

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0050.0F5D.87C4

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.17.2 255.255.255.192

!

ip default-gateway 172.16.17.1

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

logging 192.168.1.10

!

!

snmp-server community PTE RW

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

MFF-MT02SW

Current configuration : 2731 bytes

!

version 15.0

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname MFF-MT02SW

!

ip ftp username MFF-MT02SW

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name muszaki.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

ip dhcp snooping vlan 10,20,40,99

!

spanning-tree mode pvst

spanning-tree portfast default

spanning-tree portfast bpduguard default

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport access vlan 10

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 00E0.F989.C177

!

interface FastEthernet0/2

switchport access vlan 20

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0001.423B.50A1

!

interface FastEthernet0/3

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

ip arp inspection trust

ip dhcp snooping trust

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.17.3 255.255.255.192

!

ip default-gateway 172.16.17.1

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

logging 192.168.1.10

!

!

snmp-server community PTE RW

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

end

MFF-MT03SW

Current configuration : 2714 bytes

!

version 15.0

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname MFF-MT03SW

!

ip ftp username MFF-MT03SW

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name muszaki.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

ip dhcp snooping

!

spanning-tree mode pvst

spanning-tree portfast default

spanning-tree portfast bpduguard default

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport access vlan 10

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0040.0BD0.098D

!

interface FastEthernet0/2

switchport access vlan 20

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 00D0.97A6.73BB

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.17.4 255.255.255.192

!

ip default-gateway 172.16.17.1

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

logging 192.168.1.10

!

!

snmp-server community PTE RW

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

end

IFF-RTR

Building configuration...

Current configuration : 1792 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname IFF-RTR

!

login block-for 120 attempts 3 within 60

!

!

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

ip cef

ipv6 unicast-routing

!

no ipv6 cef

!

!

!

username Admin1 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

!

!

ip ftp username IFF-RTR

ip ftp password cisco123

ip ssh version 2

no ip domain-lookup

ip domain-name hbone.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0/0

description IFF-LSW1 fele

ip address 10.0.0.6 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/0/1

description PTE-RTR fele

ip address 10.0.0.2 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 10

router-id 2.2.2.2

log-adjacency-changes

network 10.0.0.0 0.0.0.3 area 0

network 10.0.0.4 0.0.0.3 area 10

!

ip classless

ip route 172.16.15.0 255.255.255.192 10.0.0.5 120

ip route 172.16.16.128 255.255.255.192 10.0.0.5 120

!

ip flow-export version 9

!

!

ip access-list extended sl_def_acl

deny tcp any any eq telnet

deny tcp any any eq www

deny tcp any any eq 22

permit tcp any any eq 22

!

no cdp run

!

banner motd ^CHBone felugyeleti eszkoz^C

!

!

!

!

line con 0

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

End

IFF-LSW1

Current configuration : 7118 bytes

!

version 16.3.2

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname IFF-LSW

!

login block-for 120 attempts 3 within 60

!

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

clock timezone PTE 1

!

!

!

!

no ip cef

ip routing

!

ipv6 unicast-routing

!

no ipv6 cef

!

!

!

username Admin1 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

!

!

!

!

ip ftp username IFF-LSW

ip ftp password cisco123

ip ssh version 2

no ip domain-lookup

ip domain-name info.ptuni.hu

!

!

spanning-tree mode pvst

!

class-map match-all TANARISUBNET

match access-group 100

!

policy-map TANAR-QOS

class TANARISUBNET

priority percent 25

set precedence 5

!

!

!

!

!

interface Port-channel1

no switchport

ip address 10.0.0.9 255.255.255.252

!

interface Port-channel2

no switchport

ip address 10.0.0.13 255.255.255.252

!

interface GigabitEthernet1/0/1

no switchport

no ip address

channel-group 2 mode active

duplex auto

speed auto

!

interface GigabitEthernet1/0/2

no switchport

no ip address

channel-group 2 mode active

duplex auto

speed auto

!

interface GigabitEthernet1/0/3

no switchport

no ip address

channel-group 1 mode desirable

duplex auto

speed auto

!

interface GigabitEthernet1/0/4

no switchport

no ip address

channel-group 1 mode desirable

duplex auto

speed auto

!

interface GigabitEthernet1/0/5

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

!

interface GigabitEthernet1/0/6

description IFF-B-1 fele

switchport access vlan 99

switchport mode access

!

interface GigabitEthernet1/0/7

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

!

interface GigabitEthernet1/0/8

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

!

interface GigabitEthernet1/0/9

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

!

interface GigabitEthernet1/0/10

!

interface GigabitEthernet1/0/11

!

interface GigabitEthernet1/0/12

!

interface GigabitEthernet1/0/13

!

interface GigabitEthernet1/0/14

!

interface GigabitEthernet1/0/15

!

interface GigabitEthernet1/0/16

!

interface GigabitEthernet1/0/17

!

interface GigabitEthernet1/0/18

!

interface GigabitEthernet1/0/19

!

interface GigabitEthernet1/0/20

!

interface GigabitEthernet1/0/21

!

interface GigabitEthernet1/0/22

!

interface GigabitEthernet1/0/23

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

switchport nonegotiate

!

interface GigabitEthernet1/0/24

no switchport

ip address 10.0.0.5 255.255.255.252

service-policy output TANAR-QOS

duplex auto

speed auto

!

interface GigabitEthernet1/1/1

!

interface GigabitEthernet1/1/2

!

interface GigabitEthernet1/1/3

!

interface GigabitEthernet1/1/4

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

mac-address 0001.c986.ae01

ip address 172.16.0.1 255.255.254.0

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

ip access-group DIAKHARDENING in

standby 1 ip 172.16.1.254

standby 1 priority 255

standby 1 preempt

!

interface Vlan20

mac-address 0001.c986.ae02

ip address 172.16.17.129 255.255.255.224

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

standby 2 ip 172.16.17.158

standby 2 priority 255

standby 2 preempt

!

interface Vlan30

mac-address 0001.c986.ae03

ip address 172.16.15.3 255.255.255.192

ip helper-address 192.168.1.2

ip helper-address 10.1.1.10

!

interface Vlan40

mac-address 0001.c986.ae04

ip address 172.16.10.1 255.255.255.0

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

ip access-group GUESTHARDENING in

standby 3 ip 172.16.10.254

standby 3 priority 255

standby 3 preempt

!

interface Vlan99

mac-address 0001.c986.ae05

ip address 172.16.16.65 255.255.255.192

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

standby 4 ip 172.16.16.126

standby 4 priority 255

standby 4 preempt

!

router ospf 20

router-id 3.3.3.3

log-adjacency-changes

passive-interface GigabitEthernet1/0/5

passive-interface GigabitEthernet1/0/6

passive-interface GigabitEthernet1/0/7

passive-interface GigabitEthernet1/0/8

passive-interface GigabitEthernet1/0/9

network 10.0.0.4 0.0.0.3 area 10

network 10.0.0.8 0.0.0.3 area 10

network 10.0.0.12 0.0.0.3 area 10

network 172.16.0.0 0.0.1.255 area 10

network 172.16.17.128 0.0.0.31 area 10

network 172.16.10.0 0.0.0.255 area 10

network 172.16.16.64 0.0.0.63 area 10

!

router rip

!

ip classless

ip route 172.16.15.0 255.255.255.192 10.0.0.10 120

ip route 172.16.15.0 255.255.255.192 10.0.0.14 130

ip route 172.16.16.128 255.255.255.192 10.0.0.10 120

ip route 172.16.16.128 255.255.255.192 10.0.0.14 130

!

ip flow-export version 9

!

!

ip access-list extended sl_def_acl

deny tcp any any eq telnet

deny tcp any any eq www

deny tcp any any eq 22

permit tcp any any eq 22

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

ip access-list extended DIAKHARDENING

remark csak tanar es management es szerver fele mehet kommunikacio, icmp echo tilt

permit ip any host 192.168.1.2

permit ip any host 192.168.1.10

deny icmp any any echo

deny ip any 172.16.15.0 0.0.0.63

deny ip any 172.16.10.0 0.0.0.255

deny ip any 172.16.11.0 0.0.0.255

deny ip any 172.16.4.0 0.0.1.255

deny ip any 172.16.17.192 0.0.0.31

deny ip any 172.16.12.0 0.0.0.255

deny ip any 172.16.8.0 0.0.1.255

deny ip any 172.16.18.0 0.0.0.31

deny ip any 172.16.14.0 0.0.0.255

permit ip any any

ip access-list extended GUESTHARDENING

remark csak szerver, management es net, szervert se pingelhet

deny icmp any any echo

permit ip any host 192.168.1.2

permit ip any host 192.168.1.10

deny ip any 172.16.0.0 0.0.1.255

deny ip any 172.16.17.128 0.0.0.31

deny ip any 172.16.2.0 0.0.1.255

deny ip any 172.16.17.160 0.0.0.31

deny ip any 172.16.15.0 0.0.0.63

deny ip any 172.16.11.0 0.0.0.255

deny ip any 172.16.4.0 0.0.1.255

deny ip any 172.16.17.192 0.0.0.31

deny ip any 172.16.12.0 0.0.0.255

deny ip any 172.16.6.0 0.0.1.255

deny ip any 172.16.17.224 0.0.0.31

deny ip any 172.16.13.0 0.0.0.255

deny ip any 172.16.8.0 0.0.1.255

deny ip any 172.16.18.0 0.0.0.31

deny ip any 172.16.14.0 0.0.0.255

permit ip any any

access-list 100 permit ip 172.16.17.128 0.0.0.31 any

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

!

!

!

snmp-server community PTE RW

!

logging 192.168.1.10

line con 0

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login

!

line aux 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

ntp authentication-key 1 md5 0822455D0A16 7

ntp authenticate

ntp trusted-key 1

ntp server 192.168.1.10

ntp update-calendar

!

end

IFF-LSW2

Current configuration : 5595 bytes

!

version 16.3.2

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname IFF-LSW2

!

login block-for 120 attempts 3 within 60

!

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

no ip cef

ip routing

!

ipv6 unicast-routing

!

no ipv6 cef

!

!

!

username Admin1 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

!

!

!

!

ip ftp username IFF-LSW2

ip ftp password cisco123

ip ssh version 2

no ip domain-lookup

ip domain-name info.ptuni.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Port-channel1

no switchport

ip address 10.0.0.10 255.255.255.252

!

interface Port-channel4

no switchport

ip address 10.0.0.17 255.255.255.252

!

interface GigabitEthernet1/0/1

no switchport

no ip address

channel-group 4 mode active

duplex auto

speed auto

!

interface GigabitEthernet1/0/2

no switchport

no ip address

channel-group 4 mode active

duplex auto

speed auto

!

interface GigabitEthernet1/0/3

no switchport

no ip address

channel-group 1 mode auto

duplex auto

speed auto

!

interface GigabitEthernet1/0/4

no switchport

no ip address

channel-group 1 mode auto

duplex auto

speed auto

!

interface GigabitEthernet1/0/5

switchport trunk native vlan 99

switchport trunk allowed vlan 20,30,99

switchport mode trunk

!

interface GigabitEthernet1/0/6

switchport trunk native vlan 99

switchport trunk allowed vlan 20,30,99

switchport mode trunk

!

interface GigabitEthernet1/0/7

switchport trunk native vlan 99

switchport trunk allowed vlan 20,30,99

switchport mode trunk

!

interface GigabitEthernet1/0/8

!

interface GigabitEthernet1/0/9

!

interface GigabitEthernet1/0/10

!

interface GigabitEthernet1/0/11

!

interface GigabitEthernet1/0/12

!

interface GigabitEthernet1/0/13

!

interface GigabitEthernet1/0/14

!

interface GigabitEthernet1/0/15

!

interface GigabitEthernet1/0/16

!

interface GigabitEthernet1/0/17

!

interface GigabitEthernet1/0/18

!

interface GigabitEthernet1/0/19

!

interface GigabitEthernet1/0/20

!

interface GigabitEthernet1/0/21

!

interface GigabitEthernet1/0/22

!

interface GigabitEthernet1/0/23

!

interface GigabitEthernet1/0/24

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,40,99

switchport mode trunk

switchport nonegotiate

!

interface GigabitEthernet1/1/1

!

interface GigabitEthernet1/1/2

!

interface GigabitEthernet1/1/3

!

interface GigabitEthernet1/1/4

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

mac-address 00d0.bc27.1e01

ip address 172.16.2.1 255.255.254.0

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

standby 5 ip 172.16.3.254

standby 5 priority 255

standby 5 preempt

!

interface Vlan20

mac-address 00d0.bc27.1e02

ip address 172.16.17.161 255.255.255.224

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

standby 6 ip 172.16.17.190

standby 6 priority 255

standby 6 preempt

!

interface Vlan30

mac-address 00d0.bc27.1e03

ip address 172.16.15.1 255.255.255.192

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

ip access-group GHHARDENING in

standby 9 ip 172.16.15.62

standby 9 priority 255

standby 9 preempt

!

interface Vlan40

mac-address 00d0.bc27.1e04

ip address 172.16.11.1 255.255.255.0

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

standby 7 ip 172.16.11.254

standby 7 priority 255

standby 7 preempt

!

interface Vlan99

mac-address 00d0.bc27.1e05

ip address 172.16.16.129 255.255.255.192

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

standby 8 ip 172.16.16.190

standby 8 priority 255

standby 8 preempt

!

router ospf 10

router-id 4.4.4.4

log-adjacency-changes

passive-interface GigabitEthernet1/0/5

passive-interface GigabitEthernet1/0/6

passive-interface GigabitEthernet1/0/7

network 172.16.2.0 0.0.1.255 area 10

network 172.16.17.160 0.0.0.31 area 10

network 172.16.11.0 0.0.0.255 area 10

network 172.16.16.128 0.0.0.63 area 10

network 172.16.15.0 0.0.0.63 area 10

network 10.0.0.8 0.0.0.3 area 10

network 10.0.0.16 0.0.0.3 area 10

!

ip classless

!

ip flow-export version 9

!

!

ip access-list extended sl_def_acl

deny tcp any any eq telnet

deny tcp any any eq www

deny tcp any any eq 22

permit tcp any any eq 22

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

ip access-list extended GHHARDENING

remark csak szerver es net fele, minden mas tilt

permit ip any host 192.168.1.2

permit ip any host 192.168.1.10

deny icmp any any echo

deny ip any 172.16.0.0 0.0.1.255

deny ip any 172.16.2.0 0.0.1.255

deny ip any 172.16.4.0 0.0.1.255

deny ip any 172.16.6.0 0.0.1.255

deny ip any 172.16.17.128 0.0.0.31

deny ip any 172.16.17.160 0.0.0.31

deny ip any 172.16.17.192 0.0.0.31

deny ip any 172.16.17.224 0.0.0.31

deny ip any 172.16.10.0 0.0.0.255

deny ip any 172.16.11.0 0.0.0.255

deny ip any 172.16.12.0 0.0.0.255

deny ip any 172.16.13.0 0.0.0.255

deny ip any 172.16.14.0 0.0.0.255

permit ip any any

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

!

!

!

snmp-server community PTE RW

!

logging 192.168.1.10

line con 0

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login

!

line aux 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

end

IFF-RSW1

Building configuration...

Current configuration : 5925 bytes

!

version 16.3.2

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname IFF-RSW

!

login block-for 120 attempts 3 within 60

!

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

no ip cef

ip routing

!

ipv6 unicast-routing

!

no ipv6 cef

!

!

!

username Admin1 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

!

!

!

!

ip ftp username IFF-RSW

ip ftp password cisco123

ip ssh version 2

no ip domain-lookup

ip domain-name info.ptuni.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Port-channel2

no switchport

ip address 10.0.0.14 255.255.255.252

!

interface Port-channel3

no switchport

ip address 10.0.0.21 255.255.255.252

!

interface GigabitEthernet1/0/1

no switchport

no ip address

channel-group 2 mode passive

duplex auto

speed auto

!

interface GigabitEthernet1/0/2

no switchport

no ip address

channel-group 2 mode passive

duplex auto

speed auto

!

interface GigabitEthernet1/0/3

no switchport

no ip address

channel-group 3 mode desirable

duplex auto

speed auto

!

interface GigabitEthernet1/0/4

no switchport

no ip address

channel-group 3 mode desirable

duplex auto

speed auto

!

interface GigabitEthernet1/0/5

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

ip access-group DIAKHARDENING in

!

interface GigabitEthernet1/0/6

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

ip access-group DIAKHARDENING in

!

interface GigabitEthernet1/0/7

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

switchport mode trunk

ip access-group DIAKHARDENING in

!

interface GigabitEthernet1/0/8

!

interface GigabitEthernet1/0/9

!

interface GigabitEthernet1/0/10

!

interface GigabitEthernet1/0/11

!

interface GigabitEthernet1/0/12

!

interface GigabitEthernet1/0/13

!

interface GigabitEthernet1/0/14

!

interface GigabitEthernet1/0/15

!

interface GigabitEthernet1/0/16

!

interface GigabitEthernet1/0/17

!

interface GigabitEthernet1/0/18

!

interface GigabitEthernet1/0/19

!

interface GigabitEthernet1/0/20

!

interface GigabitEthernet1/0/21

!

interface GigabitEthernet1/0/22

!

interface GigabitEthernet1/0/23

!

interface GigabitEthernet1/0/24

!

interface GigabitEthernet1/1/1

!

interface GigabitEthernet1/1/2

!

interface GigabitEthernet1/1/3

!

interface GigabitEthernet1/1/4

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

mac-address 0001.6331.ee01

ip address 172.16.0.2 255.255.254.0

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

ip access-group DIAKHARDENING in

standby 1 ip 172.16.1.254

!

interface Vlan20

mac-address 0001.6331.ee02

ip address 172.16.17.130 255.255.255.224

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

standby 2 ip 172.16.17.158

!

interface Vlan40

mac-address 0001.6331.ee03

ip address 172.16.10.2 255.255.255.0

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

ip access-group GUESTHARDENING in

standby 3 ip 172.16.10.254

!

interface Vlan99

mac-address 0001.6331.ee04

ip address 172.16.16.66 255.255.255.192

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

standby 4 ip 172.16.16.126

!

router ospf 10

router-id 5.5.5.5

log-adjacency-changes

passive-interface GigabitEthernet1/0/5

passive-interface GigabitEthernet1/0/6

passive-interface GigabitEthernet1/0/7

network 10.0.0.12 0.0.0.3 area 10

network 10.0.0.20 0.0.0.3 area 10

network 172.16.0.0 0.0.1.255 area 10

network 172.16.17.128 0.0.0.31 area 10

network 172.16.10.0 0.0.0.255 area 10

network 172.16.16.64 0.0.0.63 area 10

!

ip classless

ip route 172.16.15.0 255.255.255.192 10.0.0.22 120

ip route 172.16.16.128 255.255.255.192 10.0.0.22 120

!

ip flow-export version 9

!

!

ip access-list extended sl_def_acl

deny tcp any any eq telnet

deny tcp any any eq www

deny tcp any any eq 22

permit tcp any any eq 22

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

ip access-list extended DIAKHARDENING

remark csak tanar es management es szerver fele mehet kommunikacio, icmp echo tilt

permit ip any host 192.168.1.2

permit ip any host 192.168.1.10

deny icmp any any echo

deny ip any 172.16.15.0 0.0.0.63

deny ip any 172.16.10.0 0.0.0.255

deny ip any 172.16.11.0 0.0.0.255

deny ip any 172.16.4.0 0.0.1.255

deny ip any 172.16.17.192 0.0.0.31

deny ip any 172.16.12.0 0.0.0.255

deny ip any 172.16.8.0 0.0.1.255

deny ip any 172.16.18.0 0.0.0.31

deny ip any 172.16.14.0 0.0.0.255

permit ip any any

ip access-list extended GUESTHARDENING

remark csak szerver, management es net, szervert se pingelhet

deny icmp any any echo

permit ip any host 192.168.1.2

permit ip any host 192.168.1.10

deny ip any 172.16.0.0 0.0.1.255

deny ip any 172.16.17.128 0.0.0.31

deny ip any 172.16.2.0 0.0.1.255

deny ip any 172.16.17.160 0.0.0.31

deny ip any 172.16.15.0 0.0.0.63

deny ip any 172.16.11.0 0.0.0.255

deny ip any 172.16.4.0 0.0.1.255

deny ip any 172.16.17.192 0.0.0.31

deny ip any 172.16.12.0 0.0.0.255

deny ip any 172.16.6.0 0.0.1.255

deny ip any 172.16.17.224 0.0.0.31

deny ip any 172.16.13.0 0.0.0.255

deny ip any 172.16.8.0 0.0.1.255

deny ip any 172.16.18.0 0.0.0.31

deny ip any 172.16.14.0 0.0.0.255

permit ip any any

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

!

!

!

snmp-server community PTE RW

!

logging 192.168.1.10

line con 0

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login

!

line aux 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

IFF-RSW2

Current configuration : 5257 bytes

!

version 16.3.2

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname IFF-RSW2

!

login block-for 120 attempts 3 within 60

!

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

no ip cef

ip routing

!

ipv6 unicast-routing

!

no ipv6 cef

!

!

!

username Admin1 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 privilege 15 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

!

!

!

!

!

!

!

ip ftp username IFF-RSW2

ip ftp password cisco123

ip ssh version 2

no ip domain-lookup

ip domain-name info.ptuni.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Port-channel3

no switchport

ip address 10.0.0.22 255.255.255.252

!

interface Port-channel4

no switchport

ip address 10.0.0.18 255.255.255.252

!

interface GigabitEthernet1/0/1

no switchport

no ip address

channel-group 4 mode passive

duplex auto

speed auto

!

interface GigabitEthernet1/0/2

no switchport

no ip address

channel-group 4 mode passive

duplex auto

speed auto

!

interface GigabitEthernet1/0/3

no switchport

no ip address

channel-group 3 mode auto

duplex auto

speed auto

!

interface GigabitEthernet1/0/4

no switchport

no ip address

channel-group 3 mode auto

duplex auto

speed auto

!

interface GigabitEthernet1/0/5

switchport trunk native vlan 99

switchport trunk allowed vlan 20,30,99

switchport mode trunk

!

interface GigabitEthernet1/0/6

switchport trunk native vlan 99

switchport trunk allowed vlan 20,30,99

switchport mode trunk

!

interface GigabitEthernet1/0/7

switchport trunk native vlan 99

switchport trunk allowed vlan 20,30,99

switchport mode trunk

!

interface GigabitEthernet1/0/8

!

interface GigabitEthernet1/0/9

!

interface GigabitEthernet1/0/10

!

interface GigabitEthernet1/0/11

!

interface GigabitEthernet1/0/12

!

interface GigabitEthernet1/0/13

!

interface GigabitEthernet1/0/14

!

interface GigabitEthernet1/0/15

!

interface GigabitEthernet1/0/16

!

interface GigabitEthernet1/0/17

!

interface GigabitEthernet1/0/18

!

interface GigabitEthernet1/0/19

!

interface GigabitEthernet1/0/20

!

interface GigabitEthernet1/0/21

!

interface GigabitEthernet1/0/22

!

interface GigabitEthernet1/0/23

!

interface GigabitEthernet1/0/24

!

interface GigabitEthernet1/1/1

!

interface GigabitEthernet1/1/2

!

interface GigabitEthernet1/1/3

!

interface GigabitEthernet1/1/4

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

mac-address 0090.2ba7.3801

ip address 172.16.2.2 255.255.254.0

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

standby 5 ip 172.16.3.254

!

interface Vlan20

mac-address 0090.2ba7.3802

ip address 172.16.17.162 255.255.255.224

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

standby 6 ip 172.16.17.190

!

interface Vlan30

mac-address 0090.2ba7.3803

ip address 172.16.15.2 255.255.255.192

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

ip access-group GHHARDENING in

standby 9 ip 172.16.15.62

!

interface Vlan40

mac-address 0090.2ba7.3804

ip address 172.16.11.2 255.255.255.0

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

standby 7 ip 172.16.11.254

!

interface Vlan99

mac-address 0090.2ba7.3805

ip address 172.16.16.130 255.255.255.192

ip helper-address 192.168.1.2

ip helper-address 192.168.1.10

standby 8 ip 172.16.16.190

!

router ospf 10

router-id 6.6.6.6

log-adjacency-changes

passive-interface GigabitEthernet1/0/5

passive-interface GigabitEthernet1/0/6

passive-interface GigabitEthernet1/0/7

network 172.16.2.0 0.0.1.255 area 10

network 172.16.17.160 0.0.0.31 area 10

network 172.16.11.0 0.0.0.255 area 10

network 172.16.16.128 0.0.0.63 area 10

network 172.16.15.0 0.0.0.63 area 10

network 10.0.0.20 0.0.0.3 area 10

network 10.0.0.16 0.0.0.3 area 10

!

ip classless

!

ip flow-export version 9

!

!

ip access-list extended sl_def_acl

deny tcp any any eq telnet

deny tcp any any eq www

deny tcp any any eq 22

permit tcp any any eq 22

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

ip access-list extended GHHARDENING

remark csak szerver es net fele, minden mas tilt

permit ip any host 192.168.1.2

permit ip any host 192.168.1.10

deny icmp any any echo

deny ip any 172.16.0.0 0.0.1.255

deny ip any 172.16.2.0 0.0.1.255

deny ip any 172.16.4.0 0.0.1.255

deny ip any 172.16.6.0 0.0.1.255

deny ip any 172.16.17.128 0.0.0.31

deny ip any 172.16.17.160 0.0.0.31

deny ip any 172.16.17.192 0.0.0.31

deny ip any 172.16.17.224 0.0.0.31

deny ip any 172.16.10.0 0.0.0.255

deny ip any 172.16.11.0 0.0.0.255

deny ip any 172.16.12.0 0.0.0.255

deny ip any 172.16.13.0 0.0.0.255

deny ip any 172.16.14.0 0.0.0.255

permit ip any any

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

!

!

!

snmp-server community PTE RW

!

logging 192.168.1.10

line con 0

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login

!

line aux 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

IFF-GH04SW

Current configuration : 2810 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname IFF-IT04SW

!

ip ftp username IFF-IT04SW

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name info.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

ip dhcp snooping

!

spanning-tree mode pvst

spanning-tree portfast default

spanning-tree portfast bpduguard default

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport access vlan 30

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0060.5C45.81CB

!

interface FastEthernet0/2

switchport access vlan 30

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 00E0.B008.5D4E

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 20,30,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface GigabitEthernet0/2

switchport trunk native vlan 99

switchport trunk allowed vlan 20,30,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.16.130 255.255.255.192

!

ip default-gateway 172.16.16.191

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

!

!

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

IFF-GH05SW

Current configuration : 2750 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname IFF-IT05SW

!

ip ftp username IFF-IT05SW

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name info.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

ip dhcp snooping

!

spanning-tree mode pvst

spanning-tree portfast default

spanning-tree portfast bpduguard default

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport access vlan 30

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0040.0BC4.0E99

!

interface FastEthernet0/2

switchport access vlan 30

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 20,30,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface GigabitEthernet0/2

switchport trunk native vlan 99

switchport trunk allowed vlan 20,30,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.16.131 255.255.255.192

!

ip default-gateway 172.16.16.191

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

!

!

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

IFF-GH06SW

Current configuration : 2750 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname IFF-IT06SW

!

ip ftp username IFF-IT06SW

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name info.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

ip dhcp snooping

!

spanning-tree mode pvst

spanning-tree portfast default

spanning-tree portfast bpduguard default

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport access vlan 30

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0009.7C22.7767

!

interface FastEthernet0/2

switchport access vlan 30

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 20,30,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface GigabitEthernet0/2

switchport trunk native vlan 99

switchport trunk allowed vlan 20,30,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.16.132 255.255.255.192

!

ip default-gateway 172.16.16.191

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

!

!

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

IFF-IT01SW

Current configuration : 3133 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname IFF-IT01SW

!

ip ftp username IFF-IT01SW

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name info.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

ip dhcp snooping

!

spanning-tree mode pvst

spanning-tree portfast default

spanning-tree portfast bpduguard default

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport access vlan 10

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0001.9638.C556

!

interface FastEthernet0/2

switchport access vlan 20

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

!

interface FastEthernet0/3

switchport access vlan 20

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

!

interface FastEthernet0/4

switchport access vlan 20

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0006.2A54.CD8E

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface GigabitEthernet0/2

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.16.67 255.255.255.192

!

ip default-gateway 172.16.16.126

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

!

!

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

IFF-IT02SW

Current configuration : 2790 bytes

!

version 15.0

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname IFF-IT02SW

!

ip ftp username IFF-IT02SW

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name info.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

ip dhcp snooping

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport access vlan 10

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0003.E444.3C76

!

interface FastEthernet0/2

switchport access vlan 20

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0030.F2ED.9E43

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface GigabitEthernet0/2

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.16.68 255.255.255.192

!

ip default-gateway 172.16.16.126

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

logging 192.168.1.10

!

!

snmp-server community PTE RW

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

End

IFF-IT03SW

Current configuration : 2476 bytes

!

version 15.0

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname IFF-IT03SW

!

ip ftp username IFF-IT03SW

ip ftp password cisco123

enable secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name info.ptuni.hu

!

username Admin1 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

username Admin2 secret 5 $1$mERr$/VT.EosIwbboaDHzZXq7r1

!

!

ip dhcp snooping

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

switchport mode trunk

!

interface FastEthernet0/2

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface GigabitEthernet0/2

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,40,99

ip arp inspection trust

ip dhcp snooping trust

switchport mode trunk

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.16.69 255.255.255.192

!

ip default-gateway 172.16.16.126

!

no cdp run

!

banner motd ^CEz a switch a Puskas Tivadar Egyetem tulajdona. Illetektelen hozzaferes torvenyellenes!^C

logging 192.168.1.10

!

!

snmp-server community PTE RW

!

ip access-list standard vty

permit host 172.16.17.81

permit host 172.16.16.80

permit host 172.16.16.208

line con 0

password 7 0822455D0A16544541

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

line vty 5 15

access-class vty in

exec-timeout 5 0

password 7 0822455D0A16544541

logging synchronous

login local

transport input ssh

!

!

!

!

end